File save as following:

Robot movements instruction(simplified and full):

a. Joint movement: MJ, MOVJ or G0;

b. Linear movement: ML, MOVL or G1;

c. Arc movement: MC, MOVC or G6.

Storage mode of inner position data:

Before instruction MJ/L/C or MOVJ/L/C, use {} to specify position data.

Format: {I_J_K_X_Y_Z_A_B_C_U_V_x_y_z_a_b_c_}

there is no space between characters. But it’s necessary to insert space before X or x, in case of mixture leading to misread hexadecimal 0X and 0x.

Capital letter I/J/K are used to specify the middle point of arc(incremental value on the base of arc start point X/Y/Z). when movement is joint or linear, it can be omitted.

Capital letter X/Y/Z/A/B/C/U/V are used to specify the coordinates under current workpiece coordinate system and tool number.(correspond to X/Y/Z/A/B/C/Xs/Ys axis respectively ).

Lowercase x/y/z/a/b/c mean joint coordinate of J1-J6 axis(unit:1/100000 degree ), it cannot be omitted for joint movement MJ or MOVJ of 6 axis robot.

These data about position is hidden in robot, and will not display.

Attention: 1. when a point is set as middle point of arc, it workpiece posture will be ignored.

- The program generated from teaching can be edited, for example, MOVJ ca be changed into MOVL, or change MOVL into MOVJ.

Full Circle instruction:

MOVCA or MCA;

MOVC L88;

MC L88;

MOVCA L88;

MCA L88 ;

G6 L88 I_ J_ K_ X_ Y_ Z_

I_ J_ K_  means the incremental coordinates of the first middle point based on start point.(unit:mm)

X_ Y_ Z_ means the coordinates(G90 is absolute coordinate, G91 is incremental coordinate) of the second middle point based on start point.(unit:mm).

Explanation of X_Y_Z_A_B_C_ in {I_J_K_X_Y_Z_A_B_C_U_V_x_y_z_a_b_c_} .

X_Y_Z_A_B_C_ refers to position and posture of current TCP tool coordinate system in current user coordinate system., X_Y_Z_ defines position, and A_B_C_defines posture.(direction of coordinate system and rotation correspond to right-hand coordinate system principle).

It refers to that the current user coordinate system rotates by C degree around Z axis of itself., then rotates by B degree around Y axis of itself, then rotates by A degree around X axis of itself, and move the coordinate system by distance of  X_Y_Z_, finally the coordinate system coincides with current TCP tool coordinate system.

The coordinate XYZABC displayed in the user coordinate of main interface is just like above.

Program sample 1:

G56  T1

{ X1105.771Y-46.798Z381.532A10.596B-12.733C-34.748 x202362y55919z-440022a-1597120b-3913089c3917796}

MOVJ VJ=50.0

{ X1105.771Y-46.798Z283.529A10.596B-12.733C-34.748 x202362y104889z-1254487a-1804742b-4645311c4222756}

MOVL V=30.0

{ X1105.771Y-44.965Z174.524A10.596B-12.733C-34.748 x213727y7327z-2012265a-2166817b-5446662c4695017}

MOVL

{ X1091.273Y-33.025Z165.647A7.989B-13.485C-45.834 x569982y-165820z-1916944a-3464447b-4682752c6765019}

MOVL V=20.0

{I18.359J-1.26K-1.893 X1128.865Y-31.124Z158.498A-11.686B-0.337C-16.577 x496028y-1054346z-797229a-2285226b-3570802c2494242}

MOVC V=10.0

{I8.05J15.218K5.609 X1128.535Y-10.028Z160.621A-11.543B-1.952C13.994 x240642y-480245z-1444104a-704434b-4658045c-1470353}

MOVC V=10.0

{I-1.311J-11.06K-7.867 X1123.958Y-32.043Z146.116A13.919B-3.333C49.223 x-950751y-1589845z-263688a3227911b-1824048c-7140864}

MOVC V=10.0

{I10.704J-9.054K-13.566 X1139.605Y-51.541Z122.871A27.133B-6.655C89.584 x-954119y-3723536z2208264a2608756b1638090c-9936760}

MOVC V=10.0

{ X1124.576Y-51.541Z155.713A27.133B-6.655C89.584 x-965346y-3526893z2175553a2625265b1790016c-9871317}

MOVL V=20.0

{ X1092.742Y-47.174Z291.857A16.655B-9.937C63.719 x-1258787y-1699846z840000a3667838b71252c-8164161}

MOVL V=20.0

Sample 2:

G54T2

{ X418.231Y-151.032Z-93.495A-2.686B-18.605C73.239U0V0 x149197y2318883z-2486750a-280315b8966100c-5936467}

MOVJ VJ=50.0

{ X418.231Y-233.685Z-93.495A-0.516B-25.127C72.429 x-61712y2639062z-2897447a-770751b9335925c-6244530}

MOVL V=10.0

SN2

{I-37.051J39.052 X417.831Y-154.781Z-56.443A-0.516B-25.127C72.429 x504398y2020763z-2649472a-706886b9041291c-5636861}

MOVC V=10.0

{ X462.483Y-154.781Z-56.443A-0.516B-25.127C72.429 x832585y2307874z-2794519a-667934b9221432c-5329498}

MOVL